EVALUATION OF THE AMES SOLID WASTE RECOVERY SYSTEM

Section 1: Introduction

Purpose and topic of the report

Overview of report contents

Background on the power plant

Note: This report is several hundred pages, which explains the length of this introduction.

Background on the research grant and project

The following report evaluates the performance of the Ames Solid Waste Recovery System relating to the following objectives:

- Evaluation of boiler efficiency

- RDF fuel utilization

- Particulate collector efficiency

- Other boiler performance including

  - excess air flow

  - characterization of coal, RDF, grate and collector ash

  - ash-softening temperatures

  - calculation of slagging and fouling indexes

- Corrosion

Ames Solid Waste Recovery System

The Ames Solid Waste Recovery System is a continuously operating system that processes municipal solid waste (MSW) for use as a supplemental fuel in the existing steam generators of the Ames Municipal Power Plant. This system consists of a 136-Mg/day (1509-ton/day) processing plant, a 454-Mg (500-ton) Atlas storage bin, pneumatic transport systems, and the existing municipal power plant. The processing plant incorporates two stages of shredding: ferrous and nonferrous metal recovery, and an air density separator. The three steam generators consist of one pulverized coal-fired unit, two spreaders, a return traveling grate, and stoker-fired units.

EPA Research Grant

The EPA Grant No. R803903-01-0 for the first year of research on the Ames Solid Waste Recovery System was officially awarded February 4, 1976. A detailed work plan was submitted in March 1976 and included (a) environmental evaluations of steam generator units 5, 6, and 7 including particulate collectors; (b) boiler performance study; (c) boiler corrosion studies; (d) economic evaluation of the solid waste process plant and the city power plant; and (e) interim characterization of the refuse-derived fuel (RDF).

The actual studies began June 1, 1976. Because of boiler unit availability at the power plant, environmental evaluation and boiler performance of the stoker-fired units were the major research emphases.